NAME

afro - multivariate substructure factor amplitude estimation for SAD/MAD and SIRAS.

SYNOPSIS

afro hklin foo.mtz hklout foo_out.mtz

Keyworded input

DESCRIPTION

This program will determine normalized substructure factor amplitudes (ie.

E(A) values) which are needed for direct methods substructure determination

programs.  In this version, the multivariate formulas are not implemented,

but just a standard approach is taken for finding SAD or SIR differences (and

attempting to find the best wavelength to calculate the anomalous differences).

GETTING STARTED

The best way to start is to use the example scripts.

KEYWORDED INPUT

XTAL <ID>

<ID>

The crystal's name/identification string.

XTAL SUBKEYWORDS:

CELL <a> <b> <c> <alpha> <beta> <gamma>

<a> <b> <c> <alpha> <beta> <gamma>

Cell parameters for the given XTAL.

Default: take the values from the mtz file.

DNAMe <ID>

The dataset identifier.  This keyword is required.

DNAMe SUBKEYWORDS:

COLUmn F=<f> SF=<sf> F+=<f+> SF+=<sf+> F-=<f-> SF-=<sf->

Diffraction data for the XTAL and DNAMe defined.

If anomalous data is not to be used, set F and SF only.  If using

anomalous data, set F+, SF+, F-, SF-.  Setting both F and F+ will result

in an error.  If only F and DANO is present in the mtz file, use the ccp4

program mtzMADmod to change F/DANO to F+/F-.

<f>

|F| (observed structure factor amplitude *if no anomalous data is present*).

<sf>

Corresponding sigma for <f>.

<f+>

|F+| (observed structure factor amplitude of positive Bijvoet pair).

<sf+>

Corresponding sigma of <f+>.

<f->

|F-| (observed structure factor amplitude of negative Bijvoet pair).

<sf->

Corresponding sigma of <f->.

RESO <hires> <lores>

Specify resolution limits for the given XTAL and DNAMe diffraction data.

Do not use this keyword to specify a resolution cut-off for substructure detection,

but use the HIREs keyword from AFRO instead.  Only use this keyword if you wish to

remove some data that you believe to be useless.

Default: use all the data available in the mtz file.

EXCLude [SIGF <nsigf>] [SANO <nsano>] [SISO <nsiso>]

Set criteria for excluding E-values to output.  This is essential for

optimal substructure determination.  The default is to use all the data

available in the mtz file and this is not recommended for EA calculation.

The criteria and cutoff recommendations for the subkeywords are the following:

SIGF <nsigf>

exclude reflections if FP < <nsigf> * SIGF.  The recommended setting is 3.

SANO <nsano>

exclude reflections if DANO < <nsano> * SDANO.  The recommended setting is 1.

(DANO = abs(|F+| - |F-|) and SDANO is the standard deviation of DANO in measurement.

SISO <niso>

exclude reflections if abs(|Fder| - |Fnat|) < <nsiso> * SDISO.  The recommended setting is 1.

SDISO is sqrt(SIGFder^2 + SIGFnat^2).

BINS

Number of bins for luzzati parameter estimation and refinement and output of

statistics.

OPTIONAL KEYWORDS:

OUTPut <outputname>

<outputname> is the string associated with the ASCII file

that afro writes out.  The default <outputname> is "heavy".

ACENtric

Only acentric reflections will be used.  This is the default for SAD data.

HIREs <hireslimit>

Only output FA values to the following high resolution limit.  The default is

for the program to calculate it automatically.

CENTric

Only centric reflections will be used.

TITLe <title>

Title to be added to the mtz file. Default: "EA values from AFRO".

INTErpolate

Normalization is done by interpolating neighbouring bin values.

VERBose <n>

Specify amount of information to be outputted (where n is a positive integer).

n = 0 is the normal output,  n = 1 is more output and n = 2 is for debugging

purposes.  Default: n = 0.

OUTPUT:

COLUMNS in HKLOUT

*STILL TO DO*

EXAMPLES

Example 1: SAD EA value generation

#!/bin/sh

set -e

afro HKLIN $CEXAM/tutorial/data/gere_MAD_nat.mtz \

HKLOUT $CCP4_SCR/gere_SADEA.mtz << eof-afro

Xtal SeMET

DNAME PEAK

COLUmn F+=F_peak(+) SF+=SIGF_peak(+) F-=F_peak(-) SF-=SIGF_peak(-)

EXCLUDE SIGF 3 SANO 1       # don't use data with F/SF < 3 or DANO/SIGDANO < 1

RESOL HICUt 0.5             # only use data 0.5 Angstroms about hi res limit

ALLIn

eof-afro

Example 2: MAD EA value generation

#!/bin/sh

set -e

afro HKLIN $CEXAM/tutorial/data/gere_MAD_nat.mtz \

HKLOUT $CCP4_SCR/gere_MADEA.mtz << eof-afro

Xtal SeMET

DNAME PEAK

COLUmn F+=F_peak(+) SF+=SIGF_peak(+) F-=F_peak(-) SF-=SIGF_peak(-)

EXCLUDE SIGF 3 SANO 1        # don't use data with F/SF < 3 or DANO/SIGDANO < 1

RESOL HICUt 0.5              # only use data 0.5 Angstroms about hi res limit

DNAME INFL

COLUmn F+=F_infl(+) SF+=SIGF_infl(+) F-=F_infl(-) SF-=SIGF_infl(-)

EXCLUDE SIGF 3 SANO 1 SISO 1 # don't use data with F/SF < 3 or DANO/SIGDANO < 1

# or isomorphous/dispersive difference < 1

RESOL HICUt 0.5              # only use data 0.5 Angstroms about hi res limit

DNAME HIGH

COLUmn F+=F_hrm(+) SF+=SIGF_hrm(+) F-=F_hrm(-) SF-=SIGF_hrm(-)

EXCLUDE SIGF 3 SANO 1        # don't use data with F/SF < 3 or DANO/SIGDANO < 1

RESOL HICUt 0.5              # only use data 0.5 Angstroms about hi res limit

# NOTE! Afro takes only isomorphous/dispersive differences between

# the first two defined data sets!!!

# You could also define the low remote as well...

ALLIn

eof-afro

Example 3: SIRAS EA value generation

#!/bin/sh

set -e

afro HKLIN $CEXAM/tutorial/data/gere_MAD_nat.mtz \

HKLOUT $CCP4_SCR/gere_SIRASEA.mtz << eof-afro

# define the native first!!

Xtal NATIVE

DNAME NATIVE

COLUmn F=F_nat SF=SIGF_nat

EXCLUDE SIGF 3               # don't use data with F/SF < 3

RESOL HICUt 0.5              # only use data 0.5 Angstroms about hi res limit

Xtal SeMET

DNAME PEAK

COLUmn F+=F_peak(+) SF+=SIGF_peak(+) F-=F_peak(-) SF-=SIGF_peak(-)

EXCLUDE SIGF 3 SANO 1 SISO 1 # don't use data with F/SF < 3 or DANO/SIGDANO < 1

# or isomorphous difference/sigma < 1

RESOL HICUt 0.5              # only use data 0.5 Angstroms about hi res limit

ALLIn

eof-afro

Example 4: MAD + Native EA value generation

#!/bin/sh

set -e

afro HKLIN $CEXAM/tutorial/data/gere_MAD_nat.mtz \

HKLOUT $CCP4_SCR/gere_MADNATEA.mtz << eof-afro

# define the native first!!

Xtal NATIVE

DNAME NATIVE

COLUmn F=F_nat SF=SIGF_nat

EXCLUDE SIGF 3               # don't use data with F/SF < 3

RESOL HICUt 0.5              # only use data 0.5 Angstroms about hi res limit

Xtal SeMET

DNAME PEAK

COLUmn F+=F_peak(+) SF+=SIGF_peak(+) F-=F_peak(-) SF-=SIGF_peak(-)

EXCLUDE SIGF 3 SANO 1 SISO 1 # don't use data with F/SF < 3 or DANO/SIGDANO < 1

# or isomorphous difference/sigma < 1

RESOL HICUt 0.5              # only use data 0.5 Angstroms about hi res limit

DNAME INFL

COLUmn F+=F_infl(+) SF+=SIGF_infl(+) F-=F_infl(-) SF-=SIGF_infl(-)

EXCLUDE SIGF 3 SANO 1 SISO 1 # don't use data with F/SF < 3 or DANO/SIGDANO < 1

# or isomorphous/dispersive difference < 1

RESOL HICUt 0.5              # only use data 0.5 Angstroms about hi res limit

DNAME HIGH

COLUmn F+=F_hrm(+) SF+=SIGF_hrm(+) F-=F_hrm(-) SF-=SIGF_hrm(-)

EXCLUDE SIGF 3 SANO 1        # don't use data with F/SF < 3 or DANO/SIGDANO < 1

RESOL HICUt 0.5              # only use data 0.5 Angstroms about hi res limit

# NOTE! Afro takes only isomorphous/dispersive differences between

# the first two defined data sets!!!

# You could also define the low remote as well...

ALLIn

eof-afro